# BMET4790:BIOMECHATRONICS

# TUTORIAL:	Sensory Substitution Processing

Name: Hunter Tambimuttu 				SID: 510448271

Name: Caleb Nazareth 					SID: 530372811

Name: Gabriel Ralph 					SID: 470205736

Note: Activity needs to be completed and submitted before the start of the next tutorial

THIS TUTORIAL SHOULD TAKE A TOTAL OF 4 HOURS TO COMPLETE

Notes:

- Answer questions briefly and clearly within the blocks provided

- A signed plagiarism coversheet must be included with the submission

- A soft copy (including MATLAB listings) must be submitted through Canvas

- Tutorials submitted after the due date will lose 5% per day (or part thereof) unless accompanied by a valid doctor’s certificate.

- Use this document as a template for your submission

Background

The vOICe system converts an image to a sound file that can be interpreted by a blind person. It substitutes an acoustic image for a visual one using the process illustrated in Figure 1.

Figure 1: Sensory substitution from video to audio

- The resolution of the image is reduced significantly, typically to 88 pixels

- The number of grayscale shades is reduced to 3 or 4

- A composite sound signal is produced for each column by summing the sound components

  - The base frequency of each sound is scaled by the row number

  - The amplitude of each sound is weighed by the shade value of each pixel

  - The overall amplitude of the signal in each ear is weighted by the angle off the midline to produce a stereo effect

- The composite sound file for each column is played in turn to build up a complete sound “image”

Tutorial

Copy the greyscale bitmap file, face.bmp, from the Biomechatronics site on WebCT

Use the following MATLAB code to read the file

[z]=imread('face2','bmp');

zf=double(z);		% convert the image data to floating point

Question 1: Plotting the Raw Image

Note that there are various different ways that MATLAB can be used to generate a 256 shade greyscale image. If you choose not to use the process described below, please justify the alternative. Note that it may affect all of your subsequent answers.

- Use MATLAB help to find out about producing scaled images IMAGESC, making sure that the pixels are square using AXIS and using the correct colouring using COLORMAP

- Plot the raw image								(6)

- Generate axes labels using the XLABEL and YLABEL commands	(2)

- Generate a COLORBAR							(2)

- The result should look something like this

Question 2: Spatial Filtering

It is possible to sub-sample the image space to reduce the resolution to 88 by displaying 1 pixel in N. However, this loses information. A superior method is to produce a pixel which is the average of all of the surrounding pixels

- Determine the size of the original image using the SIZE command.		(2)

176x176

- To produce an 88 array, how many pixels must be skipped between samples in the x and the y directions?							(2)

Start at 11 and then take every 22nd from there.

- Use ONES to generate the basis of an N×N matrix of the correct size to average all of the cells that will be missed during the sub-sampling process. Read the help file to find out how FILTER2 uses this “kernel” to generate a 2D low-pass filter. What will the value of each element be to ensure that the average is generated? Explain your answer					(4)

The value of the kernel is 1/N where N is the number of pixels in the kernel. For N pixels, the sum of these will add to one, making it the average of the kernel region in the image.

- Use the FILTER2 command to produce an average image of the same size as the original that performs this averaging process

- Plot the result – it should look like this					(9)

Question 3: Sub-sampling

You can use FOR loops to sub-sample the image down to 88, but MATLAB has good techniques to resample the 2D image array. You should have encountered these when you worked through the introductory MATLAB tutorials

- Sub-sample the image to produce the following 8 × 8 matrix 		(9)

163.9050  179.6033  182.5847  136.0124  113.1322  191.5331  223.7355  218.2004

149.9215  164.3822   99.4050  148.4112  133.1384  102.5661  172.9607  199.7252

138.7025  108.1818   73.6178  239.6632  196.1529   80.5269   94.9773  173.4525

127.2190   52.7211  114.3636  166.3306  195.8017   87.5083   66.7934  130.8698

112.0620   46.8760  138.5310  212.4194  214.0744  124.3202   48.8202  106.2128

52.0434   27.8822   91.2066  182.4421  151.1880   89.7975   76.4236  114.3740

30.5950   21.5186   27.8450  155.0248  101.2831   48.9855   60.0723   88.2479

51.2025   22.1095   50.1942   86.4607  198.0269  176.0620  172.7314  177.2355

- Note that there is a deliberate error in one of the numbers. Highlight the correct value in your answer							(2)

- Plot the reduced resolution 8 × 8 image. It should look like this		(4)

- Use the help to find out about MAX, MIN and FLOOR to scale the image values to produce five quantised gray scale values of 0,1,2,3,4. Justify your algorithm for rounding and include the section of code that you used to perform this quantisation function						(6)

min_val = min(zf3, [], 'all'); % finds the absolute minimum

max_val = max(zf3, [], 'all'); % finds the absolute maximum

zf4 = floor(4*(zf3 - min_val)/(max_val - min_val));

% subtract the min to make zero the minimum

% divide by the range to normalise

% multiply by 4 to get four quantised values

- Plot the quantised 8 × 8 image. It should look like this			(4)

Question 4: Sensory Substitution Process

- Use LINSPACE to generate a time signal from 0 to 0.5 seconds sampled at 8kHz (it will contain 4000 samples). Include your code snippet		(2)

Fs = 8000;        % frequncy of signals

n = size(zf4, 2); % number of signals

L = 4000;         % points in signals

fl = 10*(1:n);    % frequency levels

t = linspace(0,L/Fs,L)'

- Use SIN to generate eight sinusoidal signals with frequencies corresponding to the row number (1 to 8). Starting at a base signal frequency of f1 = 10Hz ,  etc. Store these so that you can use them to build up the sound file without having to regenerate them for each column

- For each column produce a composite signal being the sum of all of the frequencies in that column, weighted by the gray scale value of each quantised pixel

- Display the normalised sound file for each column. The first should look like this										(8)

Use the FFT to confirm that the elements in the fourth column correspond to the frequencies generated in that sound file.

- List the quantised amplitudes of the pixels in the fourth column		(2)

2     2     2     x     1     3     3     3

2     2     1     x     2     1     2     3

2     1     0     x     3     1     1     2

1     0     1     x     3     0     0     2

1     0     2     x     3     1     0     1

0     0     1     x     2     1     1     1

0     0     0     x     1     0     0     1

0     0     0     x     3     2     2     2

2,2,4,2,3,2,2,1

- Plot the section of the FFT that corresponds to these spectral components	(6)

- Justify any differences in the frequency and amplitude of the spectral components									(2)

All can be seen on the plot

Question 5: Listen to the Sound File

The frequency generated starting at 10Hz is good to display the waveforms, but it is too low to be heard.

- Regenerate the sinusoidal signals:

  - First frequency = f1 = 293.66Hz (The note D4)

  - Generate a vector: f_ratios = [0     4     7    12    16    19    24    28] / 12
(For those musically inclined this is relating to semitones in a scale.
0, 12 and 24 are octaves or the tonic.
4, 16 and 28 are thirds.
7 and 19 are fifths.

  - Generate a frequency vector by raising f1 to the power of the vector.
(This contains 8 different frequencies to use for each column)

- Regenerate the 8 sinusoidal signals by the frequencies.
,  and so on.

- Regenerate the composite signals for every column

- Normalise the signal for each to lie between -1 and 1

- Split the signal into a vector with two columns to create a stereo file

- Weight the signal for each of the two stereo channels according to its offset from the centre line of the image. [7 1],[6 2],[5 3],[4 4],[4 4],[3 5],[2 6],[1 7]

- Normalise each channel again with respect to the peak value of the combined signal  so that the louder signal lies between -1 and 1

- Using the SOUND function generate the individual sounds for each column (note you will need a set of speakers or a pair of headphones to hear this)

- Verify that the sounds appear to move from the left, through the centre to the right (if your headphones are on correctly)

- Copy and paste your code underneath.

- Export and upload the file as a .wav file to Canvas

Bonus 10

Total (72+10)